# VNC Setup: Give Windows and Raspberry Pi Static IPs

Goal: Set fixed (static) IP addresses so you can reliably connect with VNC.

- Windows PC (Ethernet): 192.168.2.152 / 255.255.255.0

- Raspberry Pi (wired): 192.168.2.151 / 255.255.255.0 (/24)

- You’ll then connect in VNC Viewer to 192.168.2.151

## Part 1 — Set a Static IP on Windows (Ethernet)

- Plug in Ethernet

  - Make sure your Windows computer is connected to the Pi (directly or via a simple switch) with an Ethernet cable.

- Open Network Settings

  - Click Start → Settings (gear icon).

  - Go to Network & Internet → Ethernet (make sure you pick the wired/Ethernet adapter you’re using).

- Open IP settings

  - Scroll to IP assignment. Click Edit.

- Switch to Manual

  - In the dialog, change Automatic (DHCP) to Manual.

  - Toggle IPv4 to On (if it isn’t already).

- Enter the Static Details

  - IP address: 192.168.2.152

  - Subnet mask (if shown instead): 255.255.255.0

  - Gateway: leave blank

  - Preferred DNS: leave

- Save

  - Click Save.

  - You should briefly see the Ethernet status change and reconnect.

- Quick Check

  - Press Windows Key → type cmd → open Command Prompt.

  - Type: ipconfig and press Enter.

  - Under your Ethernet adapter, confirm IPv4 Address shows 192.168.2.152 and Subnet Mask shows 255.255.255.0.

## Part 2 — Set a Static IP on Raspberry Pi (wired)

You’ll do this first using the Pi’s current “.local” name, then switch to the static IP.

- Connect via VNC using the hostname

  - Open TigerVNC or RealVNC Viewer on Windows.

  - In the address bar, enter raspberrypi.local and connect.

  - If prompted, accept security prompts and log in.

- Open Network Connections on the Pi

  - On the Pi’s desktop, look at the top-right icons.

  - Click the two arrows (network icon) that may be blinking.

  - Choose Advanced Options → Edit Connections.

- Remove the HCU Engineering profile (if present)

  - In the list, select HCU Engineering.

  - Click the minus (–) button to remove it.

- Edit the Wired Profile

  - Select Wired connection (or similar wired profile).

  - Click Settings (or gear icon).

- Change IPv4 to Manual

  - Go to the IPv4 Settings tab.

  - Change Method from Automatic (DHCP) to Manual.

- Add the Static Address

  - Click Add (under Addresses).

  - Address: 192.168.2.151

  - Netmask: 24  (this equals 255.255.255.0)

  - Gateway: leave blank

  - DNS Servers: leave blank

  - Click Save.

- Apply and Reconnect

  - Close the network windows.

  - It may take a few seconds for the new IP to take effect.

- Close your current VNC session

  - Close the VNC window you used for raspberrypi.local.

- Reconnect using the new static IP

  - Open your VNC Viewer again.

  - In the address bar, enter 192.168.2.151 and connect.

  - Log in when prompted.

## Part 3 — Verify & Troubleshoot (quick!)

If anything doesn’t connect right away, try these in order:

- Ping test from Windows

  - Open Command Prompt and type:

ping 192.168.2.151

    - Success: You should see replies.

    - If it fails: Re-check that both devices’ IPs and subnet are exactly as above, and that Ethernet is connected.

- Confirm Windows IP again

  - ipconfig → look for IPv4 Address 192.168.2.152 and Subnet Mask 255.255.255.0.

- Confirm Pi IP locally (if you have keyboard/monitor on the Pi)

  - Open a Terminal on the Pi and run:

ip addr show eth0

    - Look for inet 192.168.2.151/24 on eth0.

- Make sure VNC is enabled on the Pi

  - On the Pi: Menu → Preferences → Raspberry Pi Configuration → Interfaces tab → VNC: Enable → OK.

  - If you had to enable it, try reconnecting in VNC after a few seconds.

- Cable/port check

  - Try another Ethernet cable or port if nothing else works.